WEATHER MINIMA

| Airspace | Flight Visibility | Distance From Cloud | Distance AGL |
| --- | --- | --- | --- |
| Control Zones and Traffic Zones | Not less than 3 SM | Horizontally: 1 SM Vertically: 500ft | Vertically: 500ft |
| Other Controlled Airspace | Not less than 3 SM | Horizontally: 1 SM Vertically: 500ft | ___________ |
| Uncontrolled 1000ft AGL or above | Not less than 1 SM (day) 3 SM (night) | Horizontally: 2000ft Vertically: 500ft | ___________ |
| Uncontrolled below 1000ft AGL | Not less than 2 SM (day) 3 SM (night) | Clear of cloud | ___________ |

SPECIAL VFR WEATHER MINIMA (CONTROL ZONE ONLY)

| Aircraft type | Flight Visibility (Ground when reported) | Distance From Cloud |
| --- | --- | --- |
| Aircraft other than helicopter | 1 SM | Clear of cloud |
| Helicopter | ½ SM | Clear of cloud |

DAY VFR INSTRUMENTS

The following instruments are required for day VFR:

- Airspeed Indicator (ASI)

- Altimeter (ALT)

- Compass

- Working timepiece

DAYLIGHT AND NIGHT

## Daylight

The period of time in any day when the center of the sun’s disc is less than 6 below the horizon, and, in any place where the sun rises and sets daily, may be considered to be the period commencing one half hour before sunrise and ending one half hour after sunset.

Night

The period of time in any day when the center of the sun’s disc is more than 6 below the horizon or the period commencing one half hour after sunset and ending one half hour before sunrise, in any place where the sun rises and sets daily.

### NIGHT VFR INSTRUMENTS

The following instruments are required for night VFR:

- Airspeed Indicator (ASI)

- A Sensitive Pressure Altimeter (ALT)

- Compass

- A Turn and Bank Indicator

- A Gyro magnetic Compass or Heading Indicator

- Means to illuminate the flight instruments

- Each crewmember must have access to a working timepiece and flashlight.

OXYGEN EQUIPMENT ORDER

No person shall fly an aircraft for more than 30 minutes at an altitude between 10,000 and 13,000 feet or above 13,000 feet unless there is oxygen available for each crewmember for two hours.

TRANSPORTATION OF DANGEROUS GOODS

Dangerous goods shall not be carried unless in accordance with the Transportation of Dangerous Goods Act.

General Prohibition:

- No person shall handle, offer for transport, transport or import any dangerous goods unless

(a) the person complies with all applicable prescribed safety requirements;

(b) the goods are accompanied by all applicable prescribed documents; and

(c) the means of containment and transport comply with all applicable prescribed safety standards and display all applicable prescribed safety marks.

- Must have permission from the Minister.

#### AER 401-04