TURBULENCE

The is an irregular motion of air resulting from "EDDIES" and vertical currents. Can cause light bumps or loss of control.

### TYPES/CAUSES OF TURBULENCE

Mechanical Turbulence (EDDIES):

Friction between the air and the ground especially from irregular terrain and man made obstacles, causes “EDDIES”

Thermal Turbulence:

Happens on hot sunny days when the sun heats the earth’s surface unevenly.  Isolated convection currents are set in motion and this results in turbulence.

Frontal Turbulence:

The lifting of the warm air by the sloping frontal surface and friction between the two opposing air masses produce turbulence in the frontal zone.

Wind Shear:

Marked changes in wind direction and speed either vertical or horizontal. If the change in wind is great you can expect severe turbulence.

## CLASSIFICATION OF TURBULENCE

Light

Momentary slight changes in altitude and/or attitude or a slight bumpiness. Occupants may feel a slight strain against their seat belts.

Moderate

Occupants will feel a definite strain against their seat belts and unsecured objects will be dislodged.

Severe

Large and abrupt changes in altitude and/or attitude and usually, large variations in indicated air speed. The airplane may momentarily be out of control.

#### Extreme

Airplane is tossed about violently and is impossible to control. May cause structural damage.

## Weather Signs

Look For Unsettled Weather When:

- barometer is falling

- temperature at night is higher is than usual

- clouds move in different directions at different levels

- cirrus clouds increase.  A large ring appears around the sun or moon and stays there until the overcast clouds thicken and obscure the sun or moon

- summer afternoon clouds darken.

##### Look For Steady Precipitation When:

- there have been signs of unsettled weather

- the wind is southeast to northeast and the pressure is falling.  Rain or snow within a day if the pressure is falling slowly.  If falling rapidly, rain or snow soon, with strengthening winds.

Look For Showers When:

- cumulus clouds develop in a westerly wind

- cumulus clouds develop rapidly in spring or summer early afternoon

Look For Clearing Weather When:

- the barometer rises

- wind shifts into west or northwest

- the temperature falls

Look For Continued Bright Weather When:

- sun sets like a ball of fire

- barometer is steady or slowly rising

- afternoon cloudiness decreases after 3PM to 4PM

- morning fog breaks within two hours after sunrise

- light breeze from west or northwest

- there is a red sunset

MET 403-12