# MAPS AND CHARTS

A map is a small-scale flat-surface representation of some portion of the earth’s surface.

A representation that is designed for plotting navigational information is called a chart.

Often times, map and chart are used interchangeably.

Because the earth is a sphere, a map must show the portion of the earth’s surface it represents with some distortion.

# Elements in Map Construction

There are four basic elements in map construction:

- Areas,

- Shapes,

- Bearings, and

- Distances.

The mathematical bases on which maps are constructed are termed projections.

Lambert Conformal Conic Projection

Concept supposes a cone superimposed over the surface of a sphere.

Properties

- Meridians of longitude converge toward the nearer pole.

- Parallels of latitude are curves which are concave toward the nearer pole.

- The scale is almost perfectly uniform.

- A straight line drawn on this map is a GREAT CIRCLE.

Mercator Projection

- Based on the fact that a cylinder has its tangency at the Equator.

- Can be visualized as  a light radiating out from the center of the earth projecting an image outwards onto the cylinder.

Properties

- Meridians of longitude are straight and parallel.

- Parallels of latitude are straight and parallel.

- There is no constant scale.

- A straight line drawn on this map is a RHUMB LINE.

- There is extreme exaggeration of longitude in northerly areas.

- Distances shown near the equator are fairly precise.

Transverse Mercator Projection

- Same technique as Mercator Projection except the cylinder is rotated 90 degrees so that point of tangency is a meridian of longitude rather than the Equator.

- Chart is accurate along the selected meridian.

## Properties

Similar to Mercator Projection except:

- Quite accurate at depicting scale.

- Distance is accurate along the meridian

- Distortion occurs at the edges of the map.

Properties of a Map

Maps have a number of properties including:

- Represents a portion of the earth.

- Has meridians of longitude and parallels of latitude.

- Scale.

- Relief

- Isogonic Lines.

- Communities, roads and railways.

- Aerodromes.

- Restricted areas.

- Compass Rose.

- Aeronautical Information.

# SCALE

Scale is a relationship between a unit of distance on the map to the distance on the earth that the unit represents. Two methods include:

Representative Fraction :

- Most common method of expressing map scale.

- Expresses the ratio of a unit of length of the map to a corresponding number of similar units on the earth.

Graduated Scale:

- A line drawn on some convenient part of the map and graduated to show the length of one mile on the map.

## RELIEF

A representation of ground elevation above sea level on aeronautical maps. There are three ways to show relief on a map:

- Layer Tinting

- Contour Lines

- Spot Heights

1). LAYER TINTING

- When the map is coloured to represent different levels of elevation.

- An elevation legend is printed on the white border of every map to show what colours are used for different elevations.

2) CONTOUR LINES

- Lines drawn on a chart joining points of equal elevation above mean sea level.

- The gradient of a slope is indicated by the horizontal distance between the contour lines.

- SPOT HEIGHTS

- When high elevations are marked by a dot with the spot height written beside the dot.

- The highest spot height in a quadrant is usually printed in larger numbers than the other spot heights.

TYPES OF AVIATION CHARTS:

VFR Navigation Charts (VNC)

- Replacing the CPC's,

- Depict more extensive geographical areas,

- Chart printed on both sides,

- Chart is identified by the name of principle landmark,

- Based on Lambert Conformal Conic Projection,

- Useful during flights at lower altitudes and slower speeds,

- Scale is 1:500,000.

World Aeronautical Charts (WAC)

- Designed for visual navigation at higher altitudes and greater speeds,

- Each chart depicts a sizeable portion of a geographical area,

- Each chart is identified by a letter and a number,

- Printed on both sides,

- Scales is 1:1,000,000.

VFR Terminal Area Charts (VTA)

- Large-scale charts (1:250,000) published for those airports in Canada which have been designated as classified airspace for control purposes,

- Based on Transverse Mercator Projection.

Enroute Charts

- Low altitude charts are for altitudes less than 18,000 feet and high altitude charts are for altitudes equal to or greater than 18,000 feet.

- Provide information for radio navigation over designated airways system …. very little visual information.

- Its scale is not consistent.

# Canada Flight Supplement

- Supplement to A.I.P. Canada.

- Lists all aerodromes shown on VNC's and WAC's.

- It also contains all kinds of valuable information related to aviation and is absolutely invaluable to a pilot.

NAV 404-03