| PROFICIENCY LEVEL ONE ROYAL CANADIAN AIR CADETS MASTER LESSON PLAN | PO: RANGE |
| --- | --- |

PO/EO:	421.04a

ENABLING OBJECTIVE:	Identify Procedures for loading, firing and unloading the Daisy 853C Air Rifle.

REFERENCE(S):	A.	CCM Cadet/CIC Daisy 853C Air Rifle Training Manual

B.	Range Standing Orders

SUPPLEMENTARY REF(S):	N/A

TRAINING AID(S):		A.	OHP

B.	OHP slides

C.	Air rifle

LEARNING AID(S):		A.	Air Rifle

B.	A-CR-CCP-266/PT-001 Level One Handbook

TEACHING POINT(S):	A. 	Procedures for Pumping

B.	Procedures for Loading

C.	Procedures for Firing

D.	Procedures for Unloading

METHOD OF INSTRUCTION:	Lecture and Demonstration

TIME:		1 x 35-minutes

| PROFICIENCY LEVEL ONE ROYAL CANADIAN AIR CADETS MASTER LESSON PLAN | PO: RANGE |
| --- | --- |

PO/EO:	421.04a

ENABLING OBJECTIVE:	Identify the procedures for loading, firing and unloading the Daisy 853C Air Rifle.

REVIEW

PO/EO:	421.03

ENABLING OBJECTIVE:		Identify the principles of marksmanship.

1.	Review weaknesses of the last EOs performance check; and/or

2.	Ask the following review question:

a.	What are the 5 Principles of Marksmanship?

Answer:	Position

Aiming

Breathing

Trigger Control

Follow-through

INTRODUCTION

WHAT:	In this lesson you will learn the procedures for pumping, loading, firing and unloading the Daisy 853C air rifle.

WHY:	Knowing the procedures for loading, firing and unloading the Daisy 853C air rifle along with the various range commands will assist in preparing the cadet for the Daisy 853C handling and range test and to participate in the familiarization practice range at the end of this PO.

WHERE:	You will use this information any time you are on a range.

| TIME | BODY | NOTES |
| --- | --- | --- |

STAGE 1	RANGE PROCEDURES

5

MINs

1.	Pumping:

OHP #1

a.	Do not pump the rifle more than once per shot. The Daisy 853C air			rifle is designed to withstand the pressure based on a single pump 			stroke. Pumping more than once may damage the pressure 				chamber and all related seals.

b.	If the air rifle is pumped more than once, or left with full chamber 			pressure for an extended period (i.e., one-hour), the compressed 			air may not expel completely upon firing. Consequently, the air rifle			may have sufficient pressure remaining in the air pressure 				chamber to fire another pellet. Therefore, a proper unload drill must			be done each time a session of firing is completed.

c.	A cadet coach may assist the shooter in pumping the rifle.

OHP #2

2.	Loading:

a.	Pick up and hold the rifle with your left hand.

b.	Ensure the safety catch is in the ON (no red) position. Following 			individual safety precautions, the rifle should already be in safe 			status with bolt open fully to the rear and pump lever partially open. 			Close the pump lever at this time.

c.	Place the sling on the rifle. Establish a good position and correct			sight picture.																					d.	Pump the rifle. This method can be done in three different ways. 															(i)	Option 1

Remove the butt from the shoulder and rest it on the mat. 				Partially open the pump lever with the right hand and return 				the right hand to the small of the butt. Grasp the pump	 lever

with your left hand halfway up the lever. Lift the rifle upwards

until the pump is fully extended while keeping your left elbow

stationary. To avoid pinching the left hand, the use of a glove

is recommended.

(ii)	Option 2

Slightly turn your body on its left side, turn the rifle sideways,				grasp the pump handle with your right hand and the 					forestock with your left hand and open the pump sideways 				until the pump handle is fully extended while keeping your 				left elbow still.

(iii)	Option 3

Coach assistance. Remove the right hand from the small of 				the butt. Hold the rifle loosely in the left hand. The coach 				should move in and pump the rifle using both hands.			e.	When the pump handle is fully extended, pause for about three 			seconds. (This is very important; if done incorrectly, the rifle will 			have insufficient air pressure).

f.	For d (i), bring the rifle down, thereby returning the pump lever to 			the closed position. For d (ii) and (iii), bring the pump lever to the 			closed position.

g.	Load the pellet or 5-shot pellet clip (the flat end faces forward).													h.	Close the bolt.

# CONFIRMATION STAGE 1

1.	Have the cadets practice the loading procedure.

STAGE 2	RANGE PROCEDURES (Continued)

20

MINs

OHP #3

1.	Firing:

a.	Place the safety catch in the OFF (red) position.																b.	Aim rifle at the target.																			c.	Squeeze the trigger.																				d.	Open the bolt.

e.	Load the rifle following the loading procedure.

f.	When firing is complete, place the safety catch in the ON (no red) 			position, and partially open the pump lever.

g.	Lay down the rifle.

OHP #4

2.	Unloading:

a.	Pick up the rifle.

b.	Remove 5-shot pellet clip (if used).

c.	Pump the rifle.

d.	Close bolt.

e.	Place the safety catch in the OFF (red) position.

f.	Aim rifle at target.

g.	Fire the action.

OHP #5

h.	Open the bolt.

j.	Place safety catch in the ON (no red) position.

k.	Open the pump lever 5-8 cm (2-3 inches).

m.	Wait to be cleared by range staff.

n.	Lay down the rifle when ordered to do so.

CONFIRMATION STAGE 2

- Have the cadets practice the procedure for firing.

2.	Have the cadets practice the procedure for unloading.

| 30 MINs | PERFORMANCE CHECK |
| --- | --- |

Test Details – 	To ensure each cadet meets the standard, administer a test individually.

The test can be multiple choice, matching, short answer or a combination

of the three. A sample test is included.

1.	Use OHP #6 and OHP #7 to test the cadets.

| 33 MINs | CONCLUSION |
| --- | --- |

SUMMARY:   	A.	In this lesson, you have learned the procedures for pumping, loading,

firing and unloading the Daisy 853C air rifle.

RE-MOTIVATION:	A.	Comment on student performance. (Identify strengths and points that

require improvement)

B.	You will apply this knowledge any time you are firing on a range.

C.	Your next lesson is PO/EO 421.04b, range commands.