ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL FOUR | PO: NAVIGATION |
| --- | --- |

- PO/EO:	417.02b

ENABLING OBJECTIVE:	Identify Navigation Charts

REFERENCE(S):	A.	A-CR-CCP-269/PH-001 Level Four Course Training Plan

Chapter 4, Pages 99-102.

B.	A-CR-CCP-269/PT-001 Level Four Handbook

Chapter 9, Section 2, Articles 14-17, Page 5 and Section 3, Articles 1-9,

Pages 1-7.

C.	A-CR-CCP-263/PT-001 From the Ground Up

Pages 174-179.

- SUPPLEMENTARY REF(S):	N/A

TRAINING AID(S):		A.	VFR Navigation Chart

# B.	Canadian Pilotage Chart (if available)

# C.	World Aeronautical Chart (if available)

# D.	VFR Terminal Area Chart (if available)

# E.	Radio Navigation Charts (if available)

LEARNING AID(S):		A.	A-CR-CCP-269/PT-001 Level Four Handbook

B.	VFR Navigation Charts

TEACHING POINT(S):	A.	Types of navigation charts

B.	Basic chart information

METHOD OF INSTRUCTION:	Lecture

TIME:		2 of 2 x 35-minutes

ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL FOUR | PO: NAVIGATION |
| --- | --- |

- PO/EO:	417.02b

ENABLING OBJECTIVE:	Identify Navigation Charts

REVIEW

- PO/EO:	417.02a

ENABLING OBJECTIVE:	Identify Navigation Charts

1.	Review weaknesses of the last EOs performance check; and/or

2.	Ask the following review questions:

- a.	What are maps used for air navigation called?

- Answer:	Charts

- b.	What are the two principle types of projection used in air navigation charts?

Answer:	(i)	Lambert Conformal Conic Projection

(ii)	Transverse Mercator Projection

INTRODUCTION

- WHAT:	In this lesson you will learn about navigation charts.

WHY:	It is necessary to be able to read a VFR navigation chart in order to plot a flight.

WHERE:	You will have an opportunity to use this knowledge during the last Navigation class, where you will plot a flight plan.

| TIME | BODY | NOTES |
| --- | --- | --- |

STAGE 1	TYPES OF NAVIGATION CHARTS

If possible, show an example of these charts.  Don’t spend

too much

time on each.

5

MINs

- 1.	Canadian Pilotage Charts (CPC):

Canadian Pilotage Charts (CPC) are designed primarily for visual

navigation and used during flights at lower altitudes and at slower speeds.

Each chart depicts a portion of Canada and is identified by the name of

the principal landmarks on the chart. CPC’s are based on the Transverse

Mercator Projection and conform to the characteristics of that projection.

OHP #1

2.	VFR Navigation Charts (VNC):

VFR Navigation Charts (VNC) are a new series of charts that will

eventually replace the Canadian Pilotage Charts.  They depict more 		extensive geographical areas.  Each VNC replaces four of the CPC series.

The chart is printed on both sides, with the northern half of the area to be

depicted on one side and the southern half on the reverse. Each chart is

identified by the name of a principle landmark. VNC’s are based on the

Lambert Conformal Projection and conform to the characteristics of that

projection. The scale of this chart is 1:500,000 or about one inch to eight

miles.

3.	World Aeronautical Charts (WAC):

World Aeronautical Charts (WAC) are designed for visual navigation and

are most useful during flight at the higher altitudes and at greater speeds.

Each chart depicts a sizeable portion of the geographical area of Canada.

Nineteen charts cover the whole country. Each chart is identified by a

letter and a number. As is the case with VFR charts, it is printed on both

sides. The title of the chart and other aeronautical information is printed on

two sides of a white side border on one edge of the map. The scale of this

chart is 1:1,000,000 or about one inch to sixteen miles.

4.	VFR Terminal Area Charts:

VFR Terminal Area Charts are large-scale charts (1: 250,000) published

for those airports in Canada which have been designated as classified

airspace for control purposes.  Before entering such classified airspace, a

VFR flight must contact the appropriate ATC unit and receive clearance to

enter the area. VTA charts depict VFR call up points outside the area and

VFR checkpoints within the area. Radio communication information and

other information that is necessary for conducting flight through the area is

given on the chart. A solid blue line marks the edge of the area. The edge

of the control zone is marked by a dashed blue line. VTA’s are based on

the Transverse Mercator Projection and conform to the characteristics of

that projection.

5.	Radio Navigation Charts:

a.	Radio Navigation Charts provide information for radio navigation			over designated airways systems. Enroute Low Altitude Charts 			(LO) are intended for use up to but not including 18,000 feet.  			Enroute High Altitude Charts (HE) are intended for use at 18,000 			feet and above. Terminal Area Charts depict aeronautical radio 			navigation information in congested areas at a larger scale and are 			intended for use form the surface up to 18,000 feet.

b.	Enroute charts do not portray any cities, towns, or topographical 			features, but depict all radio navigation aids, including airways, 			beacons, reporting points, communication frequencies, etc. Each 			chart contains two separate map sheets printed back-to-back. A 			table of symbols and a list of all radio communication facilities 			available within the area covered by the charts is printed on the 			front of the sheet.

- CONFIRMATION STAGE 1

- 1.	Name the five types of navigation charts.

Answer:	a.	VFR Navigation Chart

# b.	Canadian Pilotage Chart

# c.	World Aeronautical Chart

# d.	VFR Terminal Area Chart

# e.	Radio Navigation Chart

- 2.	What type of chart do we use when practicing our navigation skills?

Answer:	VNC

STAGE 2 	BASIC MAP INFORMATION

15

MINs

Use a VNC

to identify

this map information.

1. 	Map Scale:

The scale of a map is the ratio of a given distance on the map to the

actual distance that it represents on the earth. Scale = Map distance/Earth

distance. There are several methods of expressing scale. They are:

OHP #2

a.	Representative Fraction:

Perhaps the most common method of expressing map scale is by

representative fraction. A representative fraction expresses the

ration of a unit of length of the map to a corresponding number of

similar units on the earth. For example, a representative fraction of

1:500,000 means that one-inch on the map is equal to 500,000

inches on the earth.

b.	Statement of Words:

Another method of expressing scale is by a simple statement in

words such as "one inch represents one million inches”, which

means that one unit on the map is equivalent of one million similar

units on the earth. Other examples of scales expressed as

statements in words are:

(i)	Six-Inch Map:

This means that six inches on the map represent one mile

on the earth.

(ii)	One-Half-Inch Map:

Here one-half inch on the map represents one mile on the

earth.

(iii)	Eight-Mile Map:

This statement means that eight miles on the earth are

represented by one inch on the map.

c.	Graduated Scale:

Almost every map used in air navigation is provided with a

graduated scale, even though one of the above-noted methods of

expressing scale is also used on the map. A graduated scale is

simply a line drawn on some convenient part of the map and

graduated to show the length of one mile on the map.

2. 	Relief on Maps:

The term relief is used to describe the method of representing changes in

elevation of the earth's surface on a flat plane or map. Some of the

methods used for showing relief are listed below:

OHP #3

a.	Contour Lines:

These join points of equal height above sea level.

OHP #4

b.	Layer Tinting:

This is used between contour lines.

c.	Spot Heights:

#### These show the height above sea level of isolated obstructions and

hilltops. The greatest height on any one sheet of some map is

shown in a box.

d.	Hachures:

These are lines showing abrupt slopes. Hachures are usually used

only on Mercator Plotting Charts.

e.	Shading:

This shows abrupt slopes.

3. 	Isogonic Lines:

Isogonic lines (lines joining places of equal magnetic variation) are

depicted by dashed lines.  The degree of variation is printed at regular

intervals along the line.

4.	Latitude and Longitude:

A grid-like pattern of meridians and parallels is depicted on aeronautical

charts. The meridians are graduated in minutes of latitude and the

parallels in minutes of longitude. This makes it possible for latitude and

longitude to be measured on the chart. In addition, the subdivided

meridians provide a scale of nautical miles, since a minute of latitude may

be considered as a nautical mile.

5.	Communities, Roads and Railways:

Yellow squares represent towns and villages. Hamlets are represented by

small circles.  A city is depicted by a yellow area outlined in black that

corresponds to the actual shape and size of the community. Highways

are indicated by red or brown lines, double-lane highways by double lines.

Railways are shown as black lines.

6.	Aerodromes:

A circle represents small airports. Large airports (those with hard surface

runways) are depicted at their location on the chart by a diagram of the

landing areas. For airports with a control tower, a schematic depiction of

the airport is repeated on the white border of Canadian Pilotage Charts.

7.	Restricted Areas:

Areas over which flight is restricted are marked on aeronautical charts and

information about them is printed on the chart. These include Alert,

Danger, Restricted, Warning and Prohibited Areas, and Military Climb

Corridors.

8.	Compass Rose:

A circle overprinted on a chart divided into 360º, from which directions

may be measured, is called a compass rose. The compass rose is

oriented on the chart on the Magnetic North. It has a diameter of

approximately 30 nautical miles on Canadian Pilotage and VFR

Navigation Charts (1:500,000).

9.	Aeronautical Information:

The date of issue of a particular chart is always included with the 			information printed with the title of the chart.  A legend of aeronautical

information explaining the symbols and data appearing on the chart is

always printed somewhere on the sheet. The legend includes

topographical symbols, as well as information on aerodromes, radio

frequencies, airways and control.

- CONFIRMATION STAGE 2

- 1.	Name the three types of map scale.

Answer:	Representative fraction, statement of words and graduated				scale.

| 28 MINs | PERFORMANCE CHECK |
| --- | --- |

- Test Details - 	Each cadet will have to identify VFR navigation charts and their

- components.

1.	Test knowledge with VNC’s.

| 33 MINs | CONCLUSION |
| --- | --- |

SUMMARY:	A.	In this lesson you have learned about navigation charts.

RE-MOTIVATION:	A.	Comment on student performance. (Identify strengths and points that

require improvement)

B.	You will have an opportunity to use this knowledge during the last 				Navigation class, where you will plot a flight plan.

C.	Your next lesson is PO/EO 417.03a. In this class you will learn about navigation terms and units.