ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL THREE | PO: PROPULSION |
| --- | --- |

PO/EO:	416.01

ENABLING OBJECTIVE:	Explain Propellers.

REFERENCE(S):	A.	A-CR-CCP-268/PH-001 Level Three Course Training Plan

Chapter 4, Pages 99-102.

B.	A-CR-CCP-268/PT-001 Level Three Handbook

Chapter 9, Section 1 and 2, Articles 1-10, Pages 1-5.

C.	A-CR-CCP-263/PT-001 From the Ground Up

Pages 69-71.

SUPPLEMENTARY REF(S):	N/A

TRAINING AID(S):		A.	OHP

B.	OHP Slides

C.	Propeller (if available)

D.	Fan (if available)

# E.	Models (if available)

LEARNING AID(S):		A.	A-CR-CCP-268/PT-001 Level Three Handbook

TEACHING POINT(S):	A.	Function of a Propeller

B.	Pitch of a Propeller

C.	Types of Propellers

METHOD OF INSTRUCTION:	Lecture

TIME:		1 x 35-minutes

ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL THREE | PO: PROPULSION |
| --- | --- |

PO/EO:	416.01

ENABLING OBJECTIVE:	Explain Propellers.

REVIEW

PO/EO:	414.02

ENABLING OBJECTIVE:		Explain the production of lift.

1	A propeller, much like an aircraft’s wing, is an airfoil that produces lift.

2.	How an airfoil produces lift was covered in Level Two, Principles of Flight. Review lift

production using OHP #1.

3.	Review the weaknesses of the previous EOs performance check; and/or

4.	Answer the following review question:

a.	How does an airfoil produce lift?

Answer:	The top half is more curved than the underside. The air flowing over the

top moves faster than the air beneath. This is because it has a greater

distance to travel than the air traveling along the underside of the wing. By

applying Bernoulli’s Principle, we know that if the speed of the air

increases the pressure decreases. As a result, the air exerts less pressure

on the wing’s upper surface and the higher pressure below produces

upward lift.

INTRODUCTION

WHAT:	In this lesson you will learn about aircraft propellers.

WHY:	A basic knowledge of propellers will enhance your knowledge of propulsion systems.

WHERE:	You can apply this knowledge when visiting an aerodrome for a facilities tour, at an air-show, during glider familiarization, or even while attending summer training.

| TIME | BODY | NOTES |
| --- | --- | --- |

STAGE 1	THE FUNCTION OF A PROPELLER

5

MINs

1.	General:

a.	While most large modern airplanes are powered by jet engines,   			many small planes still benefit from the use of a good old propeller.

b.	Airplane engines have changed a great deal over the years.  The 			pioneers of aviation had to rely on fair winds to increase their speed 		above the mechanical limits of the engine.  Today, pilots can 			adjust the engine mechanics to increase its power limits.

c.	A propeller is simply a series of rotating blades, or airfoils, that are			shaped much like an aircraft wing.  The propeller is attached to the			crankshaft of the aircraft engine.  Because of their shape, the			blades develop thrust that pulls the aircraft through the air in the			same manner as propeller blades on a boat work their way through 			the water.

Supporting

Information

### In Italics

d.	Each section of a propeller is labeled. The hub is the center of the			propeller that is connected to the crankshaft. The main curved parts

OHP #2

(Supporting Information)

of a propeller, which are arranged around the hub, are called the

blades. The root is where a blade narrows and meets the hub. The

end of each blade is	 referred to as the tip.

2.	Function:

a.	The function of a propeller is to convert the turning movement of 			the crankshaft into thrust, or forward movement.

b.	The propeller is designed so that, as it rotates, it pushes air				backward causing thrust in the forward direction.

c.	A propeller blade is an airfoil section, similar to the airfoil section			of a wing. When a propeller starts rotating, it creates the same			forces that are created when air flows over an aircraft wing: lift and			drag.

d.	However, in the case of a propeller, these forces are designated as 			thrust and torque. Furthermore, while a wing creates lift in an 			upward direction, a propeller creates thrust in a forward direction.

e.	Propellers that are attached forward of the engine and that pull from 		the front of the aircraft are called tractors. Those that are attached

aft of the engine and push from behind are called pushers.

3.	Pitch:

12

MINs

a.	In order to obtain this forward pull, or thrust, the propeller blade 			must be set at a certain angle to its plane of rotation in the same 			manner that a horizontal wing must be set at an angle to its forward 		path.

OHP #3

OHP #4

b.	The pitch is the forward distance traveled by the propeller in one			revolution. It is determined by the blade angle.

c.	When a propeller blade is set at a large angle, it will travel forward			a greater distance per revolution, therefore moving the aircraft			faster. This position is called coarse pitch. It is much like high 			gear in a car. It is used for high-speed cruise and high altitude 			flight.

d.	When a propeller blade is set at a small angle, it will travel forward 			less distance per revolution, thereby enabling the engine to develop 		greater power. This position is called fine pitch. It is much like low

gear in a car. It is used for take-off and climbs.

CONFIRMATION STAGE 1

1.	How does a propeller work?

Answer:	A propeller blade is an airfoil section, similar to the airfoil				section of a wing.  When a propeller starts rotating, it creates 			the same forces that are created when air flows over an 				aircraft wing: lift and drag.  However, in the case of a 					propeller, these forces are designated as thrust and torque.

2.	What is pitch?

Answer:	The pitch is the forward distance traveled by the propeller in				one revolution.

STAGE 2 	TYPES OF PROPELLERS

20

MINs

1.	Fixed Pitch Propeller:

OHP #5

A fixed pitch propeller has one-piece construction and only one pitch

setting.  Since the fixed pitch propeller cannot be adjusted to meet the

varying power requirements encountered during take-off, cruise and high

altitude flying, a compromise pitch setting is necessary.  Accordingly,

cruising power is chosen as being the most desirable setting for maximum

efficiency. Because of their limitations, fixed pitch propellers are only

found on low-powered, light aircraft.

2.	Adjustable Pitch Propeller:

An adjustable pitch propeller is a variation of the fixed pitch propeller and

is subject to similar limitations. The blades can be adjusted on the ground,

when the engine is stopped, to increase 	the performance during a

particular condition of flight. However, since the setting is constant and

cannot be changed in flight, maximum propeller efficiency is possible only

at the selected setting.

3.	Controllable Pitch Propeller:

A controllable pitch propeller has two pitch settings that may be selected

in flight. The fine pitch is used when maximum power is required for take-

off. The coarse pitch is used for high-speed cruise and high altitude flight.

4.	Constant Speed Propeller:

The constant speed propeller is a controllable pitch propeller with a	governor unit added. This enables it to adjust itself automatically to 	flight

and power variations. Thus, with a constant speed propeller it is possible

to obtain full power from the engine at any altitude or flight condition.

CONFIRMATION STAGE 2

1.	What are the four types of propellers?

Answer:	The fixed pitch, adjustable pitch, controllable pitch and 				constant speed propellers.

| 28 MINs | PERFORMANCE CHECK |
| --- | --- |

Test Details - 	Each cadet will have to explain how a propeller functions and identify the 				different types.

1.	Test knowledge with OHPs.

| 33 MINs | CONCLUSION |
| --- | --- |

SUMMARY:	A.	In this lesson you have learned about aircraft propellers.

RE-MOTIVATION:	A.	Comment on student performance. (Identify strengths and points that

require improvement)

B.	You can apply this knowledge when visiting an aerodrome for a facilities		tour, at an air-show, during glider familiarization, or even while attending 	summer training.

C.	Your next lesson is PO/EO 416.02. In this class you will learn about aircraft engine instruments.