ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL THREE | PO: NAVIGATION |
| --- | --- |

PO/EO:	417.04b

ENABLING OBJECTIVE:	Indicate the direction of a plane.

REFERENCE(S):	A.	A-CR-CCP-268/PH-001 Level Three Course Training Plan

Chapter 4, Pages 125-128.

B.	A-CR-CCP-268/PT-001 Level Three Handbook

Chapter 10, Section 5, Articles 24-27, Pages 7-9.

C.	A-CR-CCP-263/PT-001 From the Ground Up

Pages 166.

SUPPLEMENTARY REF(S):	N/A

TRAINING AID(S):		A.	OHP

B.	OHP Slides

C.	VFR Navigation Charts (VNC)

D.	Navigation Protractor or Plotter

LEARNING AID(S):		A.	A-CR-CCP-268/PT-001 Level Three Handbook

B.	VFR Navigation Charts (VNC)

C.	Navigation Protractor or Plotter

D.	Rulers, Pencils and Erasers

TEACHING POINT(S):	A.	The compass rose

B.	Magnetic and True North

C.	Meridian lines

D.	Direction

METHOD OF INSTRUCTION:	Lecture and Performance

TIME:		2 of 2 x 35-minutes

ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL THREE | PO: NAVIGATION |
| --- | --- |

PO/EO:	417.04b

ENABLING OBJECTIVE:	Indicate the direction of a plane.

REVIEW

PO/EO:	417.04a

ENABLING OBJECTIVE:	Indicate the position of a plane.

1.	Review weaknesses of the last EOs performance check; and/or

2.	Ask the following review questions:

a.	What is the Prime Meridian?

Answer:	The Prime Meridian is one of the two lines by which grid references are 				calculated. It is the meridian of longitude that is numbered 0 and passes 				through Greenwich England.

b.	How are both latitude and longitude measured?

Answer:	Latitude and longitude are both expressed in degrees, minutes and seconds.

INTRODUCTION

WHAT:	In this lesson you will learn how to indicate an aircraft’s direction.

WHY:	Navigation is an integral part of air training. Understanding how to identify an aircraft’s direction is a fundamental part of navigation.

WHERE:	You will apply this knowledge as you continue to learn about Navigation. By the end of Level Four Navigation you will be able to plot a flight plan all by yourself.

| TIME | BODY | NOTES |
| --- | --- | --- |

STAGE 1	DETERMINING DIRECTION

5

MINs

1.	General:

a.	The term direction can be defined as “the path pursued by a

OHP # 1

moving body” or “the point to which one moves or at which one 			looks.” Direction is an essential part of navigation and there is a 			method to determining and expressing it.

b.	Direction is measured clockwise from north to the nearest degree 			and is expressed as a three-figure group. Degrees are defined on 			the compass rose, which is divided clockwise from north into 360 			degrees. North is given 000 (often 360 is used) and therefore, east,

OHP # 2

which is 90 degrees from north is 090, south is 180 and west is

270.

2.	Using the Map:

a.	When planning a flight, pilots draw a line between the departure 			point and the expected arrival point.  They then calculate the angle 			this line makes with a meridian of longitude.  The angle is always 			measured clockwise from the particular chosen meridian using a 			compass rose.

15

MINs

b.	The angle between the meridian over which an aircraft is flying and			the line in which the aircraft’s nose is pointing, measured clockwise, 		is the true heading.

Demonstrate how to determine direction

using a VNC and either a protractor or

a plotter.

c.	It is best to measure this angle using a protractor or plotter.

3.	The Protractor:

a.	A protractor, being square, can be used both for determining 			direction and as a straightedge. The instrument has a compass 			rose graduated in 360º marked around the outer edges. It is 				transparent so that, when placed on a map, the map is visible 			through it.

OHP # 3

b.	Place the protractor on the map with the hole in the centre lying on 			the track at a point where the north-south line on the protractor lies 			along a meridian. If this is not convenient, the lines may be lined 			up parallel with the nearest meridian. The direction is read off 			where the track line 	crosses the edge of the protractor. Make sure 			you read the protractor on the side closest to the destination point.

4.	The Plotter:

OHP # 4

a.	A plotter combines a protractor and a straightedge in one device. 			It also incorporates a mileage scale for both 1: 500,000 and 1: 			1,000,000 charts. The plotter is made of clear plastic so that details

of the chart can be seen through it.

b.	With the straightedge, the pilot can draw a track from the airport of 			departure to the planned destination. The direction of a track is 			determined by using the protractor portion of the plotter. It is

numbered from 000º to 180º on the outside scale and from 190º to 			360º on the inside scale. The outside scale is for easterly tracks 			and the inside scale for westerly tracks.

c.	To use the plotter, place the hole in the centre of the plotter over an 		intersection of the track line and one of the longitude lines on the 			chart.  A point somewhere near the mid point of the track is best 			chosen to obtain greater accuracy.

d.	Place a pencil point through the hole and rotate the plotter until the 			top edge of the straightedge is aligned with the track line. Read the

direction of the track in degrees true where the longitude line of the 			chart intersects the scales. (Use the outer scale for easterly tracks 			and the inner scale for westerly tracks.)

5.	True Direction:

a.	The geographical North Pole, know as true north, is the reference 			point for measurement of a true direction. The direction of true			north never changes and therefore all maps and charts use true 			north as a datum.

b.	Unfortunately, there is no convenient method of measuring true 			north directly and continuously. Therefore, when navigating in the 			air, pilots use magnetic direction.

c.	When planning a cross-country flight, we would first determine 			the desired true direction using a navigation chart and plotter or 			protractor. However, we need a corresponding magnetic direction

to follow in the aircraft. From the navigation chart we can find the

local variation, the difference between the two directions, in order to

determine the required magnetic direction.

d.	Furthermore, when en route and we want to know our true

direction, we have to first use a compass to find our magnetic 			direction. If we know the variation, the difference between the two, 			we can then derive the true direction.

e.	Remember that true direction must be derived from a magnetic			direction, and vise versa.

# CONFIRMATION STAGE 1

1.	What is a compass rose?

Answer:	Degrees are defined on the compass rose, which is divided 				clockwise from north into 360 degrees.

1.	How do can we derive our true direction?

Answer:	From our magnetic direction.

| 28 MINs | PERFORMANCE CHECK |
| --- | --- |

Test Details - 	Each cadet will have to indicate an aircraft’s direction

1.	Test knowledge with a VNC and a protractor or plotter.

| 33 MINs | CONCLUSION |
| --- | --- |

SUMMARY:	A.	In this lesson you have learned about indicating direction.

RE-MOTIVATION:	A.	Comment on student performance. (Identify strengths and points that

require improvement)

B.	You will apply this knowledge as you continue to learn about Navigation.  			By the end of Level Four Navigation you will be able to plot a flight plan all 			by yourself.

C.	This is the last class of PO 417. Next period will be your Navigation test.